SURGE SYSTEM

STAND MOUNTED

SUGGESTED SPECIFICATION

Specifier Note:  to use as a project specification;

A. Insert, in the blank spaces provided, the applicable information

B. Insert the appropriate information from the selections provided in “[ ]”.

C. Delete the items in parentheses or marked "►" which are not applicable to the project requirements.

D. Insert, where applicable, optional non-standard features desired.

SPECIFICATION

Furnish one (1) stand mounted surge system, as supplied by the boiler manufacturer, model _____________ consisting of a _______ gallon ( [steel], [galvanized] or [epoxy lined] ) receiver, bolted on a structural steel stand with integral supports.  Receiver shall be equipped with all necessary openings for makeup, returns, overflow, vent, drain and pump suction openings.

Standard trim includes water level gauge glass with shut off valves, dial thermometer, make-up water valve, and high/low water alarm package.

Transfer pumps shall be vertical centrifugal multi-stage diffuser type with ( [bronze] or [stainless steel] ) impellers, diffusers and seals rated for 250°F.  Pumps to be capable of being removed from the system without disturbing the discharge piping.  The pumps will be capable of _____ gpm at ____ psig and will be coupled with a ____ hp, TEFC, 3500 rpm, ( [208], [240] or [460] ) /3/60 volt motor.

The pump(s) shall completely wired to a NEMA 12 panel that is to include magnetic starter(s) with overload(s), light(s), H-O-A selector switch(es) and terminal strip.